Asymmetric Effects of Stability-Inducing Operators across Trajectory and Population Search in Job-Shop Rescheduling

Takumi Kito

Department of Industrial and Management Systems Engineering,

Graduate School of Creative Science and Engineering, Waseda University, Tokyo, Japan

Tel: (+81) 80-4756-3741, Email: kito@toki.waseda.jp

Abstract. In predictive-reactive rescheduling, a high-quality pre-disruption schedule Sp already exists, and revised schedules must be both efficient (makespan) and stable (small deviation from Sp). Filling the vicinity of Sp (the high-stability region) thus becomes a performance axis as important as global search quality. Through controlled comparisons sharing an identical N5 local search, we show that a trajectory-based method (ILS) fills this vicinity by itself through continuous transformation, whereas a population-based method (memetic algorithm) remains structurally coarse there because of crossover (H1); ILS thereby wins high-stability coverage in all eight scenarios with complete separation (|δ|=1.0). We further propose a rescheduling-specific adaptation of path relinking whose guiding solution is fixed to Sp (PR), and its truncated variant (repair), as stability-inducing operators that pull solutions toward Sp. Experiments over eight scenarios, seven methods, and ten trials show that the same operator acts asymmetrically depending on the host structure—more than doubling the population's high-stability coverage and lifting it to first place in overall quality, while the trajectory host is already saturated (H2)—and quantify a complementary structure in which the best method switches with the evaluation metric.

Keywords: Rescheduling; Job-shop scheduling; Stability; Iterated local search; Path relinking

1. INTRODUCTION

Manufacturing shop floors frequently face disruptions—operation delays, machine breakdowns—that make the original schedule infeasible. Of the two response types—static (built-in tolerance) and dynamic (after-the-fact revision)—we address the latter, specifically predictive-reactive rescheduling (Ouelhadj and Petrovic, 2009), which repairs the executing schedule into a feasible one after a disruption. We focus on operation delays.

The efficiency of the revised schedule (makespan, MS) and the amount of change from the pre-disruption schedule Sp (stability) trade off. Large changes incur shop-floor confusion, setup and fixture changeovers, material re-kitting, worker re-assignment, and rippling schedule changes to downstream or outsourced operations, so stability is an objective on par with MS (Wu et al., 1993; Rangsaritratsamee et al., 2004); we formulate multi-objective optimization of efficiency and stability.

The essential peculiarity of rescheduling is that a high-quality pre-disruption schedule Sp already exists as an initial solution, and the revised schedule must deviate little from it. Optimal solutions are likely distributed near Sp, so the evaluation of a search method gains a new axis: not mere global search ability, but how well it fills the vicinity of Sp (the high-stability region).

Our aim is to analyze, under controlled conditions, how the rescheduling-specific existence of a high-quality initial solution Sp reshapes the behavior of the two metaheuristic families—trajectory and population. We also propose stability-inducing operators (PR and repair) that actively pull solutions toward Sp, and verify the whole comparison under a multi-perspective evaluation methodology integrating Pareto coverage, performance by stability band, and speed. Our focus is the structural analysis of this operator–host interaction rather than a performance race against the state of the art (Sörensen, 2015).

2. RELATED WORK AND POSITIONING

Joint consideration of efficiency and stability. The JSSP is NP-hard and metaheuristics are the mainstream (Ouelhadj and Petrovic, 2009). Joint optimization of efficiency and stability in rescheduling was pioneered by Wu et al. (1993) and pursued for the JSSP by Rangsaritratsamee et al. (2004) (hybrid GA) and Zhang et al. (2013) (GA + tabu search), and for flow shops by Katragjini et al. (2013). These works mostly bundle the two objectives into one weighted-sum scalar (Rangsaritratsamee et al., 2004; Zhang et al., 2013), where stability enters not as an independent mechanism but passively, as a single evaluation term of the objective. Of the two metaheuristic families—trajectory-based (single-solution) and population-based (Blum and Roli, 2003)—the solvers here are predominantly population-based, anchored in GAs.

Embedding stability as a mechanism (scope-limiting approaches). A separate lineage guarantees stability structurally by limiting the rescheduling scope: match-up scheduling (Bean et al., 1991), AOR, which reschedules only the affected operations (Abumaizar and Svestka, 1997), Zakaria and Petrovic (2012), who restrict the chromosome to an interval, and Sun et al. (2026), who formalize the scope into a four-level hierarchy. Common to all is guaranteeing stability by restricting the search space.

Remaining issues and positioning. Three issues remain. First, existing rescheduling studies are predominantly GA-based (population) and lack a controlled contrast with a trajectory-based method; to our knowledge, how the rescheduling-specific existence of a high-quality initial solution Sp affects search-structure behavior (trajectory vs. population) has not been analyzed head-on; we do so via a controlled contrast between ILS and a memetic algorithm sharing the same local search (N5). Second, neither existing means of securing stability actively steers the search toward it: scope limiting excludes trade-off solutions outside the restriction, and objective embedding leaves high-stability filling to the raw ability of the base search. We propose operators (PR, repair) that actively pull solutions toward Sp; securing stability as an operator rather than by restricting the space is our core design claim against the scope-limiting family. Third, existing evaluations are mostly scalar comparisons at a few fixed weights, lacking a multi-axis methodology covering Pareto coverage, stability bands, and convergence speed; we evaluate with union HV, high-stability HV, and AOC (Section 3.4).

We pose two hypotheses. H1 (suitability): a population-based method (memetic) that recombines solutions wholesale by crossover scatters offspring away from Sp and fills the high-stability region only coarsely, whereas a trajectory-based method (ILS) that fills the neighborhood by continuous transformation from Sp covers it efficiently. H2 (complementarity): operators pulling solutions toward Sp (PR, repair) complement precisely the neighborhood filling that populations are structurally poor at, so their effect appears asymmetrically with the host structure (large for the population, marginal for the trajectory).

3. PROBLEM SETTING AND PROPOSED METHODS

3.1 Problem Definition

We target operation delays because they leave the operation set unchanged and can be handled by resequencing alone, so the revised schedule stays near Sp—the premise on which rescheduling most cleanly rests. After a single operation delay (length Δ) in an n-job, m-machine JSSP, we seek a revised schedule Sq given the original Sp. The reference is the right-shift solution SRSR—the schedule the shop floor follows by keeping the machine sequences of Sp and absorbing the delay. Operations started before the rescheduling time tr (when the delay is resolved) are frozen; those from tr onward are the optimization targets. The decision variables are only the per-machine sequences of the target operations; a solution becomes an executable schedule by fixing the frozen part and shifting each operation to its earliest feasible start after tr under the given machine sequences.

Stability metric. Deviation measures fall into two families: (i) start-time deviation (temporal stability) and (ii) processing-sequence (permutation) deviation (sequence stability). Sun et al. (2026), for example, distinguish the two explicitly and adopt the former (the sum of absolute start-time deviations, ADST). We adopt the latter because our search operators (the swap moves of PR/repair, the N5 neighborhood) are all permutation operations and it is highly independent of MS.

D(Sp,Sq)=(i,j)∈Ooptri,jp−ri,jq	(1)

Here ri,j is the processing rank of job j on machine i, and Oopt is the set of target operations (from tr onward). D=0 corresponds to the sequence-preserving right-shift solution SRSR. Temporal stability based on start times is outside this metric's scope—an explicit limitation.

Multi-objective formulation and scalarization. We minimize (MS(Sq), D(Sp, Sq)) jointly via a weighted sum with weight λ∈[0,1].

F(Sq)=λD(Sp,Sq)+(1−λ)MS(Sq)	(2)

Hatted symbols denote min–max normalized values. We sweep the weight over multiple points, merge the solutions, and evaluate Pareto coverage (Section 3.4).

Why weighted scalarization rather than Pareto-native methods. First, consistency with operational practice: the efficiency–stability priority is specified in advance as a weight by the decision maker (the standard rescheduling framework (Rangsaritratsamee et al., 2004)); each weight yields a solution, aligning with the per-weight anytime HV(t) (AOC, Section 3.4) that measures preference-specific ramp-up. Second, a common basis for controlled comparison: only by having ILS and the memetic share the same scalar objective F(λ) can we mount the same operator on both structures unmodified and isolate structural differences (methods premised on population-specific non-dominated sorting, e.g., NSGA-II, do not fit this control). To avoid dependence on any single weight, we sweep λ and merge all non-dominated solutions under UEA (Ishibuchi et al., 2020). The weighted sum misses concave parts of the front (Marler and Arora, 2010), but because all methods are evaluated under identical conditions, the structural discussion is unaffected.

3.2 Base Search Structures: ILS and Memetic-LS (H1)

In rescheduling—where the initial solution Sp is high-quality and optima lie near it—a trajectory (single-solution) method that continuously transforms from Sp should fill the high-stability region better (H1). We therefore center the comparison on ILS vs. Memetic-LS with the local search (N5) aligned, removing the local-search confound and contrasting only the search structure.

Among single-solution methods we adopt ILS (Lourenço et al., 2019) because it cleanly separates exploitation (local search) from escape (perturbation). This separation lets the perturbation strength directly control the distance from Sp and lets the stability-inducing operator (Section 3.3) slot in naturally as a perturbation. The local search is the N5 neighborhood standard in the JSSP literature (Nowicki and Smutnicki, 1996), shared by ILS and Memetic-LS: it restricts candidates to adjacent-job swaps at critical-block ends (only moves with a prospect of makespan improvement) but accepts them by their F(λ) value. This adjacent swap is a minimal structural change, so it fills the high-stability region in fine increments of deviation from Sp, fitting our stability-focused setting. The perturbation is an insert move whose strength (= distance from Sp) cycles in a sawtooth with stagnation (VNS-type (Mladenović and Hansen, 1997)). The acceptance criterion is better acceptance: the local optimum obtained after a perturbation replaces the current one only when it strictly improves the best F(λ).

Population-based methods (GA / Memetic-LS). The population-based counterpart (the control arm for H1) encodes individuals in the operation-based representation (Bierwirth, 1995) (chromosomes decoded by the GT algorithm (Giffler and Thompson, 1960)) with a standard GA (PPX crossover (Bierwirth et al., 1996), inversion mutation, tournament selection with elitism), applying the same N5 to each individual in a Lamarckian manner to form Memetic-LS (GA + local-search integration (Neri and Cotta, 2012)); the plain GA is a reference baseline. Sharing N5 with ILS, Memetic-LS differs from it solely in the search structure (single trajectory vs. population + crossover). Crossover is a destructive operation splicing two parents, so offspring readily jump away from the vicinity of Sp; even Memetic-LS with N5 therefore fills the high-stability region only coarsely—the crux of H1.

Figure 1: Schematic search behavior on the (MS, D) plane. (a) population scatters away from Sp via crossover; (b) trajectory (ILS) fills the vicinity of Sp; (c) PR/repair pulls scattered solutions back toward Sp.

3.3 Stability-Inducing Operators (PR and repair) and H2

Path relinking (PR). PR exploits the idea that good intermediate solutions lie on the path connecting two high-quality solutions; it is commonly combined with scatter search, linking solutions within an elite pool (Glover et al., 2000; Peng et al., 2015). Our rescheduling-specific feature is that the guiding solution is fixed to the single point Sp, the pre-disruption schedule: from the current local optimum (initiating) toward Sp (guiding), the path is traced by swap moves—each picks a position where the current per-machine sequence disagrees with Sp and swaps in the operation Sp has there, advancing one step toward Sp—shrinking the disagreements one by one, returning the best solution on the path. Fixing to Sp (a) makes PR uniquely interpretable as a "directed move toward the stability anchor," Sp being the optimal endpoint of the stability objective, and (b) lines the intermediates between Scur, a local optimum of F(λ), and Sp up along the MS–stability trade-off, contributing directly to Pareto filling. Each step picks one feasible disagreement swap at random, keeping evaluations at O(d) (d = number of disagreements; no significant HV difference from best-selection O(d2) in preliminary experiments).

Stability repair kick (repair). A mini-PR truncated midway: a few such swap moves pull the solution toward Sp ("repairing" the stability lost by drifting), and the local search re-optimizes from the pulled-back solution. Varying the depth (the number of swaps) covers the stability side of the front in a band.

Mounting on the hosts. On the trajectory host the operators serve as the perturbation of ILS: they fire upon stagnation, with the repair depth cycling in a sawtooth (Section 3.2). On the population host they are embedded in each individual's refinement: after the local-search pass the operator is applied stochastically to the individual, with the depth drawn at random up to the disagreement count, and the outcome, re-optimized by N5, returns to the population unconditionally, leaving acceptance to tournament selection. What both hosts mount is the identical operator—swap moves toward Sp; the hosts differ only in how it is fired.

H2 (complementarity hypothesis). PR and repair both pull solutions toward Sp, complementing exactly the high-stability filling that population-based search is poor at; ILS has already filled the neighborhood by itself (H1), leaving little headroom. The mechanism lies in solution redundancy and path length: GA-derived solutions are not MS-optimized as rigorously as N5 achieves—leaving redundant sequences whose stability can improve without hurting MS—and sit far from Sp with long PR paths, while ILS clings to the vicinity of Sp with short paths. Hence we predict an asymmetry: large effects on the population, marginal on the trajectory (Figure 1).

We study seven methods: ILS-baseline / ILS+repair / ILS+PR / GA / Memetic-LS / Memetic+PR / Memetic+repair.

3.4 Evaluation Framework

Because a single-weight scalar comparison is weight-dependent and blind to trade-off coverage, we sweep λ (weighted-sum sweep) and measure Pareto coverage under UEA (Ishibuchi et al., 2020), which archives all non-dominated solutions visited. Rescheduling also values stable solutions near Sp over the efficiency extreme, and anytime availability of a good solution matters operationally. Three metrics answer the three questions—overall quality, filling with stable solutions, speed: union HV (overall quality) = hypervolume over the whole region; high-stability HV (primary) = hypervolume restricted to D < P50 (near Sp), P50 being the median D of Pareto solutions pooled over all methods and trials; AOC (anytime performance (López-Ibáñez and Stützle, 2014)) = the time average of the HV-versus-log-time curve. Since the P50 threshold is pooled over the compared methods, we swept it over P25–P75: no high-stability comparison in Section 4 reverses (the population-side operator gains hold at p=0.001 at every threshold; H1's complete separation holds over P33–P67); only orderings among the top methods shift.

For cross-scenario comparison, HV is computed after affine normalization to [0,1]2 per scenario with reference point (1.1,1.1). The anytime HV(t) for AOC is measured on the wall clock, with the integration window common to all methods and the same logarithmic-width normalization (apples-to-apples). AOC averages the anytime HV(t) over the 10-weight sweep.

Statistics. Within each scenario we use the one-sided Wilcoxon signed-rank test (paired by trial; one-sided since H1 and H2 fix the direction in advance) plus Cliff's δ. When |δ|=1.0 (complete separation), the one-sided Wilcoxon with n=10 reaches its lower bound p ≈ 0.001, so "p=0.001" below signifies complete directional consistency (magnitudes are reported via δ and ratios). Cross-scenario tendencies are summarized by Friedman tests, average ranks, and Kendall's W; since the eight scenarios derive from five instances and the la36 ladder and ta21 pair vary only the disruption within one instance, these cross statistics are an exploratory summary of rank consistency, not independent-sample tests. Magnitudes are also given as ARPD% ((best − x)/best × 100; lower is better).

4. COMPUTATIONAL EXPERIMENTS

This section establishes H1 (Section 4.2, the trajectory's suitability for the high-stability region) and H2 (Section 4.3, the host-dependent asymmetric effect of the same operator) through controlled pairwise comparisons, then surveys their implications—a complementary structure in which the best method changes with the evaluation axis—with an overall scoreboard (Section 4.4).

4.1 Experimental Setup

Benchmarks: mt10, la21, la36, la40, ta21. Disruptions total 8 scenarios: la36S/la36M/la36L (27/54/73%), ta21S/ta21L (32/82%), mt10 (72%), la21 (35%), la40 (32%), where the percentage is the rescheduling ratio ρ=nres/ops—the fraction of all operations that became re-optimization targets Oopt—which we use as the control axis. In particular, the la36 ladder (27/54/73%) and the ta21 pair (32/82%) vary only the rescheduling ratio stepwise within the same instance and the same Sp, isolating the ρ-dependence of method differences without confounds. Each disruption is a one-operation completion delay with Δ=60–148 (≈0.9–1.5× the affected operation's processing time), and tr is its delay-resolution time (Section 3.1). Complete per-scenario definitions are in the public repository (https://github.com/kitotakumi/stability_scheduling).

Weights and trials. λ is swept over 10 points (0–0.9, step 0.1; the pure-stability endpoint λ=1.0 is excluded since the optimum degenerates to the trivial SRSR (D=0)); 10 trials per (scenario × method). The unit of analysis is one trial (the whole 10-weight sweep), yielding one HV per trial (reported values are medians over trials unless noted); all tests use n=10.

Computational budget. ILS iteration cap 3000; GA/memetic 500 generations (both budgets large enough for natural convergence). Iterations and generations differ in unit, so nominal values are aligned within each family and cross-method speed is compared by wall-clock AOC. Wall-clock time differs across families (Memetic+PR is 5–8× ILS), but a convergence check over all seven methods shows the median run reaching 99% of its final HV within half its budget—so the gap is not a truncation artifact.

Initial solution Sp. Sp is a high-quality schedule generated by GA-500. All methods receive Sp as a common start (ILS as the initial solution; GA/memetic within the initial population).

Search-structure settings (population: GA/Memetic; trajectory: ILS). The population (GA/Memetic) fixes the GA backbone (cxpb=0.85, mutpb=0.1, pop=50) at standard values, not an independent variable. The trajectory (ILS) cycles the insert perturbation strength (number of insert moves) over 2–5 in a sawtooth (Section 3.2).

Stability-inducing operator (PR/repair) settings. Firing conditions differ by host. On the population host the operator fires with probability 0.3 per individual, its depth drawn at random up to the disagreement count (for PR, N5 re-optimizes the top-3 path intermediates). On the trajectory host it fires as a perturbation upon stagnation—thresholded at 400 stagnant iterations for the first kick (letting the initial descent converge) and 10 thereafter (PR uses k=1, the single best intermediate; the repair depth sweeps the full path length—the disagreement count between the current solution and Sp—in a sawtooth). All values are common to the eight scenarios, not tuned per instance.

Environment and reproducibility. AMD Ryzen 5 7530U; Python 3.12 (NumPy, DEAP, SciPy). All seeds fixed. Code is available in the same repository.

4.2 Result 1: Trajectory (ILS) vs. Population (Memetic) (H1)

To avoid operator confounds, we compare ILS-baseline and Memetic-LS with the local search aligned. The high-stability gap is therefore attributable to the fact that even a competent population with the same local search cannot structurally fill the vicinity of Sp because of crossover. Figure 2 shows the results.

• Union HV: comparable (ILS 5 wins, Memetic 3). Memetic wins precisely on the scenarios with large improvement headroom at mid-to-high rescheduling ratios (la36M, mt10, la36L; 54–73%), the winner switching with the ratio (Section 4.4).

• High-stability HV (primary): ILS wins in all 8 scenarios with complete separation (p=0.001, |δ|=1.0). In the three low-ratio scenarios (la36S, ta21S, la40), Memetic-LS places not a single solution in the high-stability region—only ILS covers the vicinity of Sp; in the remaining five, ILS is 2–4.5×. With |δ|=1.0, significance survives Holm correction over the 8-scenario family everywhere.

• AOC: ILS significantly ahead in 6/8 (exceptions: la36S and mt10).

Figure 2: H1—(a) union HV is comparable, but (b) high-stability HV favors ILS in all 8 scenarios, and (c) AOC favors ILS in 6/8 (ILS-baseline vs. Memetic-LS, medians; *p<.05 **p<.01 ***p<.001).

Structural cause (visit-density difference map, Figure 3). The per-method normalized visit-density difference on an (MS,D) grid shows ILS concentrating on the low-D front band while Memetic disperses into the high-D unstable region. The population does visit low D, but destructive crossover keeps it from refining MS there as thoroughly as the continuously transforming ILS, even with the same N5; at equal low D its solutions lose to ILS in MS and fail to fill the region in the Pareto sense—the structural cause of the high-stability HV gap. Holding many solutions far from Sp also lengthens the paths, creating exactly the headroom the operators (PR/repair) can fill by pulling back—picked up in H2 (Section 4.3).

Figure 3: Structural cause of H1—normalized visit-density difference maps (red: ILS-baseline denser; blue: Memetic-LS denser; sign·√¦Δ¦); orange: ILS PF; green: Memetic PF. Four representative scenarios; the rest are in the repository.

4.3 Result 2: Asymmetric Effects of the PR/repair Operators (H2)

We examine the high-stability HV gain from adding the operators to each baseline, by host (Figure 4).

• Population (Memetic): substantial improvement (more than 2× in all 8 scenarios, p=0.001, |δ|=1.0). The operators directly fill the previously unreachable vicinity of Sp; the population catches up with ILS in the high-stability region and overtakes it in overall quality (union HV) (Section 4.4).

• Trajectory (ILS): mostly saturated. In most scenarios the baseline has already filled the high-stability region, tying at the ceiling. The operators are significant only in the highest-ratio band: la36L (73%) and ta21L (82%) (raw p=0.016, 0.001 for PR and repair alike; after Holm correction over the 8-scenario ILS family only ta21L remains significant, padj≈0.008; the gain is small—the ta21L median rises 0.029 → 0.031—but consistently positive across all trials). The effect is thus not absent: ILS's own filling leaves little headroom; a small margin remains only when the disruption is too large for the filling to keep up.

Figure 4: H2—the operators (PR, repair) (a) more than double the population's high-stability HV in every scenario, but (b) are mostly saturated on the trajectory host (high-stability HV medians vs. baseline; *p<.05 **p<.01 ***p<.001).

Mechanistic cause (PR path statistics, Figure 5). The path statistics directly corroborate the asymmetry: Memetic has large path lengths d0 and finds improving solutions on roughly 30–65% of paths, whereas ILS has short paths and a discovery rate near 0% everywhere (0.4% even on ta21L). This reflects more than trial opportunities: ILS has already filled the vicinity of Sp—the short paths are themselves a symptom—so no headroom remains on the segments PR traverses, whereas in Memetic many unoptimized intermediates remain between the scattered solutions and Sp; the directed move thus comes up empty on ILS. The slight la36L/ta21L effect owes to the post-kick local search re-optimizing pockets that ILS's own filling left unfilled, not to the directing itself.

Figure 5: Mechanistic cause of H2—(a) PR path length d0 (mean disagreements to Sp) and (b) improvement discovery rate (share of calls finding a solution better than the initiating one). Memetic: long paths, many improvements; ILS: short paths, almost none.

PR or repair—host-dependent usage. Whether PR and repair need distinguishing—on quality (union/high-stability HV) and anytime performance (AOC)—depends on the host. On ILS (small d) they are nearly indistinguishable: quality is saturated with either (significant only on la36L/ta21L), and—the operator fires only upon stagnation (and not in the early phase, which the log-time AOC weights heavily) with short, cheap O(d) paths—the AOC differences among ILS-baseline/+repair/+PR are non-significant in all 8 scenarios (effectively tied); the choice is immaterial. On Memetic (large d) the division is clear: PR is slightly better in quality, while repair beats PR in AOC in all 8 scenarios (δ up to +1.00)—PR traverses the long path to Sp before returning the best intermediate, ramping up slowly, whereas repair truncates after a few moves and updates the incumbent immediately. Hence: Memetic+PR when final quality is paramount, repair when the budget is tight and anytime performance matters.

4.4 Overall Scoreboard and Integration of Results

We survey the structures established by H1 and H2 on the overall scoreboard of 7 methods × 8 scenarios × 3 metrics (Figure 6), corroborating the complementary structure announced at the outset, in which the best method switches with the metric. Friedman average ranks separate the methods clearly on all three metrics, with medium-to-large cross-scenario rank consistency (Kendall's W=0.59/0.81/0.63, p<0.0001; per Section 3.4, an exploratory summary given the correlated scenarios).

Reading the metrics. Reading the green (best) distribution metric by metric makes the complementary structure concrete (parenthetical figures are Friedman average ranks, lower is better). On union HV (quality), the operator-equipped populations form the top group (Memetic+PR 2.0, Memetic+repair 2.5), the three ILS variants and the plain Memetic-LS sit in the middle (3.6–4.8), and only GA trails (6.9); the method that tied with ILS in Section 4.2 was the plain population (Memetic-LS), and the operator-equipped Memetic+PR rises above it to lead—robust to leave-one-out (unchanged whichever scenario is removed), so the narrow margin is not driven by any single scenario. On high-stability HV (primary), the ILS family and the operator-equipped Memetic cluster at the top (2.6–3.4), while only the operator-less populations collapse with ARPD ≈ 70–78%, failing to reach the vicinity of Sp (GA 6.4, Memetic-LS 6.6)—exactly what H1 (the trajectory fills it natively) and H2 (the operators complement the population's coarseness so it catches up) predict. On AOC (anytime), the three ILS variants lead (2.5–2.8), the plain Memetic-LS and Memetic+repair are in the middle (3.5, 4.0), and the slow-starting Memetic+PR and GA trail (5.6, 7.0). The best method changes with the evaluation axis (Memetic+PR for overall quality; the ILS family for speed; high stability shared by ILS and the operator-equipped Memetic), and the two structures complement the different demands of rescheduling.

Figure 6: Overall scoreboard—(a) union HV, (b) high-stability HV, (c) AOC over 7 methods × 8 scenarios. Cells: RPD% relative to the best of each scenario (green = best, 0%); each panel sorted by its own Friedman average rank. The right-most column reports ARPD% as mean/median across scenarios.

[Exploratory observation] Union-HV winner and the rescheduling ratio. The union HV winner corresponds to ρ, splitting at around 50%: ILS below, Memetic at mid-to-high (on the same la36 ladder 27/54/73%, ILS → Memetic → Memetic); only ρ, which incorporates the fixed-skeleton proportion, captures this split—the number of movable operations or the problem size does not. Our reading: the stability term makes Sp (D=0) an attractor. At small ρ the fixed skeleton dominates the solution, so no good local optimum far from Sp can arise however the movable part is reordered; good solutions are thus confined to the Sp neighborhood, and ILS's neighborhood filling suffices. As ρ grows and the efficiency extreme recedes, the single trajectory struggles to escape the basin while Memetic, retaining crossover-scattered individuals, reaches that distant region (isomorphic to H1)—ρ thus proxies the difficulty for the trajectory of escaping the attractor. However, ta21L (82%) is an exception (≈tie, p=0.053), and confounding by Sp quality and dependence on the permutation-deviation representation remain, so this stays exploratory; the primary high-stability dominance and operator asymmetry are established independently by direct tests on all 8 scenarios.

Divergent vs. convergent. ILS (spreading outward from Sp) and Memetic+PR (pulling a scattered population toward Sp) reach similar final Pareto quality from opposite directions, but ILS holds a good incumbent early while Memetic+PR ramps up after a warm-up; AOC aggregates this crossing on a log-time axis, reflecting early performance—hence the ILS family's clear lead there.

5. CONCLUSIONS

Targeting stability-aware JSSP rescheduling, this study compared trajectory vs. population search structures, proposed the stability-inducing operators (PR, repair), and built a multi-perspective evaluation methodology, verified over 8 scenarios × 7 methods × n=10. The claims are threefold.

1. The trajectory-based method (ILS) efficiently fills the high-stability region (H1). Comparable to the population in overall quality, in the primary high-stability region it wins in all 8 scenarios with complete separation over the counterpart sharing the identical N5 (p=0.001, |δ|=1.0; 2–4.5× in five scenarios, the population reaching the region not at all in the other three) and leads in anytime performance in 6/8—derived from the search structure, not the presence of local search.

2. The effects of PR and repair appear asymmetrically with the host structure (H2). They more than double the population's high-stability HV, lifting it to the ILS level, whereas the trajectory, having already filled the neighborhood, is mostly saturated. The asymmetry is not fixed, however: at extreme rescheduling ratios where ILS's filling cannot keep up (ta21L, 82%), a significant effect remains even on the trajectory.

3. Complementary structure of trajectory and population. The best method changes with the metric: overall quality (union HV) favors Memetic + mechanism, high stability is shared by ILS and Memetic + mechanism, and speed (anytime) favors the ILS family. Two options avoid the bottom group on all three metrics: Memetic+repair (mid-tier in anytime) when overall quality matters more, the ILS family (mid-tier in union HV) when speed does. Only when squeezing out the last of final quality, at the cost of a slow ramp-up, does Memetic+PR edge ahead. The two structures thus complement the different demands of rescheduling (Section 4.3).

Implementing the stability lever as an "operator" made it portable to both hosts, and isolating its host-dependent asymmetry under control is the central contribution of this study.

Limitations. (i) Results rest on n=10 (saturated main conclusions are robust; borderline cases less so). (ii) The rescheduling-ratio–union-HV-winner correspondence involves confounding by Sp quality and dependence on the stability representation, and stays exploratory. (iii) Stability is measured only by permutation deviation; validity under start-time (temporal) deviation is unverified (Section 3.1).

Future work. Re-verification under start-time deviation (temporal stability); extension to disruptions altering machine assignments (e.g., breakdowns); integrating the scope lever with the operator lever (the inducing operators within the affected range); comparison with an Sp-biased crossover as an alternative population-side prescription (a direct test of the crossover-destructiveness reading of H1); and re-verification with Pareto-native methods (e.g., NSGA-II).

REFERENCES

Abumaizar, R. J. and Svestka, J. A. (1997) Rescheduling job shops under random disruptions. International Journal of Production Research, 35, 2065-2082.

Bean, J. C., Birge, J. R., Mittenthal, J., and Noon, C. E. (1991) Matchup scheduling with multiple resources, release dates and disruptions. Operations Research, 39, 470-483.

Bierwirth, C. (1995) A generalized permutation approach to job shop scheduling with genetic algorithms. OR Spektrum, 17, 87-92.

Bierwirth, C., Mattfeld, D. C., and Kopfer, H. (1996) On permutation representations for scheduling problems. Parallel Problem Solving from Nature—PPSN IV, LNCS 1141, Springer, 310-318.

Blum, C. and Roli, A. (2003) Metaheuristics in combinatorial optimization: overview and conceptual comparison. ACM Computing Surveys, 35, 268-308.

Giffler, B. and Thompson, G. L. (1960) Algorithms for solving production-scheduling problems. Operations Research, 8, 487-503.

Glover, F., Laguna, M., and Martí, R. (2000) Fundamentals of scatter search and path relinking. Control and Cybernetics, 29, 653-684.

Ishibuchi, H., Pang, L. M., and Shang, K. (2020) A new framework of evolutionary multi-objective algorithms with an unbounded external archive. Proceedings of the 24th European Conference on Artificial Intelligence (ECAI 2020), IOS Press, 283-290.

Katragjini, K., Vallada, E., and Ruiz, R. (2013) Flow shop rescheduling under different types of disruption. International Journal of Production Research, 51, 780-797.

López-Ibáñez, M. and Stützle, T. (2014) Automatically improving the anytime behaviour of optimisation algorithms. European Journal of Operational Research, 235, 569-582.

Lourenço, H. R., Martin, O. C., and Stützle, T. (2019) Iterated local search: framework and applications. In Gendreau, M. and Potvin, J.-Y. (eds.), Handbook of Metaheuristics, 3rd ed., Springer, 129-168.

Marler, R. T. and Arora, J. S. (2010) The weighted sum method for multi-objective optimization: new insights. Structural and Multidisciplinary Optimization, 41, 853-862.

Mladenović, N. and Hansen, P. (1997) Variable neighborhood search. Computers & Operations Research, 24, 1097-1100.

Neri, F. and Cotta, C. (2012) Memetic algorithms and memetic computing optimization: A literature review. Swarm and Evolutionary Computation, 2, 1-14.

Nowicki, E. and Smutnicki, C. (1996) A fast taboo search algorithm for the job shop problem. Management Science, 42, 797-813.

Ouelhadj, D. and Petrovic, S. (2009) A survey of dynamic scheduling in manufacturing systems. Journal of Scheduling, 12, 417-431.

Peng, B., Lü, Z., and Cheng, T. C. E. (2015) A tabu search/path relinking algorithm to solve the job shop scheduling problem. Computers & Operations Research, 53, 154-164.

Rangsaritratsamee, R., Ferrell Jr, W. G., and Kurz, M. B. (2004) Dynamic rescheduling that simultaneously considers efficiency and stability. Computers & Industrial Engineering, 46, 1-15.

Sörensen, K. (2015) Metaheuristics—the metaphor exposed. International Transactions in Operational Research, 22, 3-18.

Sun, R., Cheng, G., Ding, Q., and Zhao, X. (2026) Impact of optimization scope on solution quality and stability in dynamic flexible job shop rescheduling. Computers & Industrial Engineering, 215, Article 111943.

Wu, S. D., Storer, R. H., and Chang, P.-C. (1993) One-machine rescheduling heuristics with efficiency and stability as criteria. Computers & Operations Research, 20, 1-14.

Zakaria, Z. and Petrovic, S. (2012) Genetic algorithms for match-up rescheduling of the flexible manufacturing systems. Computers & Industrial Engineering, 62, 670-686.

Zhang, L., Gao, L., and Li, X. (2013) A hybrid genetic algorithm and tabu search for a multi-objective dynamic job shop scheduling problem. International Journal of Production Research, 51, 3516-3531.